CV ASSIGNMENT

1. write program to display image matrix.

for row in image_matrix:

row_str = ""

for pixel in row:

if pixel == 0:

row_str += " . "  # Print a dot for 0 (black)

else:

row_str += " # "  # Print a hash for 255 (white)

print(row_str)

# Example image matrix

image_matrix = [

[0, 0, 0, 0, 0],

[0, 255, 255, 255, 0],

[0, 255, 255, 255, 0],

[0, 255, 255, 255, 0],

[0, 0, 0, 0, 0]

]

2. write program to display Image Histogram

# Calculate the histogram

def calculate_histogram(image):

histogram = [0] * 256

for row in image:

for pixel in row:

histogram[pixel] += 1

return histogram

# Display the histogram

def display_histogram(histogram):

for i, count in enumerate(histogram):

print(f"Intensity {i}: {'*' * count}")

# Example usage

image_histogram = calculate_histogram(image_matrix)

display_histogram(image_histogram)

# Example image matrix

image_matrix = [

[0, 0, 0, 0, 0],

[0, 255, 255, 255, 0],

[0, 255, 255, 255, 0],

[0, 255, 255, 255, 0],

[0, 0, 0, 0, 0]

]

3.write program to find 1st order derivative and display image and first order derivative

# Example image matrix

image_matrix = [

[10, 20, 30, 40, 50],

[60, 70, 80, 90, 100],

[110, 120, 130, 140, 150],

[160, 170, 180, 190, 200],

[210, 220, 230, 240, 250]

]

# Calculate the first-order derivative in the x-direction:

def first_order_derivative_x(image):

derivative_x = [[0 for _ in range(len(image[0]))] for _ in range(len(image))]

for i in range(len(image)):

for j in range(1, len(image[0])):

derivative_x[i][j] = image[i][j] - image[i][j-1]

return derivative_x

# Calculate the first-order derivative in the y-direction:

def first_order_derivative_y(image):

derivative_y = [[0 for _ in range(len(image[0]))] for _ in range(len(image))]

for j in range(len(image[0])):

for i in range(1, len(image)):

derivative_y[i][j] = image[i][j] - image[i-1][j]

return derivative_y

# Display the derivative matrices

def display_derivative(derivative_matrix):

for row in derivative_matrix:

row_str = ""

for pixel in row:

row_str += f"{int(pixel):<5}"

print(row_str)

# Example usage

derivative_x_matrix = first_order_derivative_x(image_matrix)

print("First Order Derivative in X-Direction:")

display_derivative(derivative_x_matrix)

derivative_y_matrix = first_order_derivative_y(image_matrix)

print("\nFirst Order Derivative in Y-Direction:")

display_derivative(derivative_y_matrix)

4. write program to find second order derivative and display image and second order derivative

# Example image matrix

image_matrix = [

[10, 20, 30, 40, 50],

[60, 70, 80, 90, 100],

[110, 120, 130, 140, 150],

[160, 170, 180, 190, 200],

[210, 220, 230, 240, 250]

]

# Calculate the second-order derivative in the x-direction

def second_order_derivative_x(image):

derivative_x = [[0 for _ in range(len(image[0]))] for _ in range(len(image))]

for i in range(len(image)):

for j in range(1, len(image[0])-1):

derivative_x[i][j] = image[i][j+1] - 2 * image[i][j] + image[i][j-1]

return derivative_x

# Calculate the second-order derivative in the y-direction

def second_order_derivative_y(image):

derivative_y = [[0 for _ in range(len(image[0]))] for _ in range(len(image))]

for j in range(len(image[0])):

for i in range(1, len(image)-1):

derivative_y[i][j] = image[i+1][j] - 2 * image[i][j] + image[i-1][j]

return derivative_y

# Display the derivative matrices

def display_derivative(derivative_matrix):

for row in derivative_matrix:

row_str = ""

for pixel in row:

row_str += f"{int(pixel):<5}"

print(row_str)

# Example usage

derivative_x_matrix = second_order_derivative_x(image_matrix)

print("Second Order Derivative in X-Direction:")

display_derivative(derivative_x_matrix)

derivative_y_matrix = second_order_derivative_y(image_matrix)

print("\nSecond Order Derivative in Y-Direction:")

display_derivative(derivative_y_matrix)

5.write a program for Image enhancement using Fourier transformation:

# Example grayscale image matrix

image_matrix = [

[10, 20, 30, 40, 50],

[60, 70, 80, 90, 100],

[110, 120, 130, 140, 150],

[160, 170, 180, 190, 200],

[210, 220, 230, 240, 250]

]

# Discrete Fourier Transform (DFT) calculation

def dft2D(image):

M = len(image)

N = len(image[0])

dft_result = [[0 for _ in range(N)] for _ in range(M)]

for u in range(M):

for v in range(N):

sum_val = 0

for x in range(M):

for y in range(N):

pixel_val = image[x][y]

term = pixel_val * (complex((2 * 3.1415 * (u * x / M + v * y / N))).exp())

sum_val += term

dft_result[u][v] = sum_val

return dft_result

# Inverse Discrete Fourier Transform (IDFT) calculation

def idft2D(dft_result):

M = len(dft_result)

N = len(dft_result[0])

idft_image = [[0 for _ in range(N)] for _ in range(M)]

for x in range(M):

for y in range(N):

sum_val = 0

for u in range(M):

for v in range(N):

dft_val = dft_result[u][v]

term = dft_val * (complex((-2 * 3.1415 * (u * x / M + v * y / N))).exp())

sum_val += term

idft_image[x][y] = int(sum_val.real / (M * N))

return idft_image

# Example usage

dft_result = dft2D(image_matrix)

enhanced_image_matrix = idft2D(dft_result)

# Display enhanced image

for row in enhanced_image_matrix:

print(row)

6. write program to smooth image using gaussian filter and display image

def convolve2D(image, kernel):

m, n = kernel.shape

y, x = image.shape

y = y - m + 1

x = x - m + 1

new_image = np.zeros((y, x))

for i in range(y):

for j in range(x):

new_image[i][j] = np.sum(image[i:i+m, j:j+m]*kernel)

return new_image

def gaussian_kernel(size, sigma=1):

size = int(size) // 2

x, y = np.mgrid[-size:size+1, -size:size+1]

normal = 1 / (2.0 * np.pi * sigma**2)

kernel = np.exp(-((x**2 + y**2) / (2.0*sigma**2))) * normal

return kernel

# Implementing Gaussian Filter

def gaussian_filter(image, kernel_size, sigma):

kernel = gaussian_kernel(kernel_size, sigma)

return convolve2D(image, kernel)

# Example Usage

# Assume 'image' is your grayscale image data

smoothed_image = gaussian_filter(image, 3, 1)  # 3x3 kernel size with sigma = 1

7. write program to find Histogram equalization and display that image

# Example image matrix

image_matrix = [

[10, 20, 30, 40, 50],

[60, 70, 80, 90, 100],

[110, 120, 130, 140, 150],

[160, 170, 180, 190, 200],

[210, 220, 230, 240, 250]

]

# Calculate histogram

def calculate_histogram(image):

histogram = [0] * 256

for row in image:

for pixel in row:

histogram[pixel] += 1

return histogram

# Perform histogram equalization

def histogram_equalization(image, image_histogram):

total_pixels = sum(image_histogram)

cum_histogram = [sum(image_histogram[:i+1]) for i in range(len(image_histogram))]

equalized_image = [[0 for _ in range(len(row))] for row in image]

for i in range(len(image)):

for j in range(len(image[i])):

equalized_image[i][j] = 255 * cum_histogram[image[i][j]] / total_pixels

return equalized_image

# Display the image

def display_image(image):

for row in image:

row_str = ""

for pixel in row:

row_str += f"{int(pixel):<4}"

print(row_str)

# Example usage

image_histogram = calculate_histogram(image_matrix)

equalized_image = histogram_equalization(image_matrix, image_histogram)

display_image(equalized_image)

8. write program to determine the edges using connected set V. Draw line to draw an object border

# Example image matrix

image_matrix = [

[0, 0, 0, 0, 0],

[0, 1, 1, 1, 0],

[0, 1, 1, 1, 0],

[0, 1, 1, 1, 0],

[0, 0, 0, 0, 0]

]

# Function to determine the edges using the connected set V

def connected_set_v(image):

edges = set()

rows = len(image)

cols = len(image[0])

# Check each pixel for edge condition

for i in range(rows):

for j in range(cols):

if image[i][j] == 1:

if i == 0 or i == rows - 1 or j == 0 or j == cols - 1:

edges.add((i, j))

else:

if image[i-1][j] == 0 or image[i+1][j] == 0 or image[i][j-1] == 0 or image[i][j+1] == 0:

edges.add((i, j))

return edges

# Function to display the object border using lines

def display_object_border(edges):

max_row = max(edges, key=lambda x: x[0])[0]

max_col = max(edges, key=lambda x: x[1])[1]

for row in range(max_row + 1):

line = ""

for col in range(max_col + 1):

if (row, col) in edges:

line += "# "

else:

line += ". "

print(line)

# Example usage

edges = connected_set_v(image_matrix)

display_object_border(edges)